GI Bronch – Scope not advancing (master triage + resolutions)

Access Level: 1

Date: 2025-10-26

Keywords: scope not advancing, not reaching, stuck, jammed, GI scope, colonoscope, duodenoscope, Bronch scope, bronchoscope, entrance, mark, 0–12, 2, 3, 4, 6, 8, extender, cushion, obstruction, extension limiter

LLM operator block (follow exactly):

Goal

- Handle “scope not advancing / not reaching / stuck” for the GI-Bronch simulator in one conversation.

- Ask at most the necessary questions. If the user already provided the information, skip the question and route directly.

- After choosing a section, OUTPUT ONLY that section’s **Resolution** steps (and any brief Recognition line), not the whole document.

Stage 1 — Determine scope type

- If the message does not specify the scope type, ask ONCE:

“Which scope are you using — a GI scope (colonoscope/duodenoscope) or a Bronch scope?”

- If “GI/colonoscope/duodenoscope” → proceed to Stage 2 (mark triage).

- If “Bronch/bronchoscope” → go to section [S-BRONCH] and use its Resolution.

Stage 2 — GI mark triage (only for GI scopes)

- If the message does not include a mark number, ask ONCE:

“At what mark number on the scope does it stop (0–12)? If it’s a range, reply like ‘2–3’ or ‘6–8’.”

- Routing after the answer:

* If ANY reported mark ≤ 3 (including ranges like 3–4) → choose section [S-GI-ENTRANCE].

* If ANY reported mark is 4–12 → choose section [S-GI-EXTENDER].

- If the user already provided both scope type and mark, do NOT ask; route immediately.

Ambiguity handling

- If still ambiguous after the single follow-up, briefly show BOTH GI resolutions (one-paragraph summaries) and ask which matches best.

- Keep replies concise, procedural, and ordered. Never mix steps from different sections.

Examples (for the LLM)

- “scope not advancing” → Ask scope type.

- “gi scope not advancing” → Ask mark.

- “colonoscope stops at 2–3” → Use [S-GI-ENTRANCE].

- “not advancing past 5” → Use [S-GI-EXTENDER].

- “bronch scope not advancing” → Use [S-BRONCH].

Routing cheat-sheet

1) GI scope → mark triage → (≤3 → [S-GI-ENTRANCE]) (4–12 → [S-GI-EXTENDER])

2) Bronch scope → [S-BRONCH]

------------------------------------------------------------

[S-GI-EXTENDER]  GI scope not reaching / not advancing all the way in (Extender engaged)

Triage marks: 4–12 (middle)

Recognition:

- Scope advances to mid-range (e.g., marks 6–8) then meets a firm stop or cannot pass a consistent depth.

Resolution:

1) Verify the extender / extension limiter on the left side is fully retracted.

2) If still blocked: open a service ticket and include serial number, scope model, and the exact mark where it stops.

------------------------------------------------------------

[S-GI-ENTRANCE]  GI scope stuck at entrance (marks 2–3) — Cushion / obstruction

Triage marks: ≤3 (entrance)

Recognition:

- Scope stops near the entrance (marks ~2–3) with higher-than-normal resistance.

Resolution:

1) Inspect the entrance cushion/seal for damage or folding; reseat or replace if needed.

2) Verify insertion path alignment; center the angulation wheels before insertion so the tip is straight.

3) Remove any debris at the insertion sleeve/port; apply a small amount of baby powder.

5) If resistance persists, stop and open a service ticket with serial number, scope model, and the exact mark where it stops.

------------------------------------------------------------

[S-BRONCH]  Bronch scope not advancing — placeholder

Scope type: Bronch scope (bronchoscope); uses a separate airway entrance.

Resolution (current status):

- Inserting Bronch scope, by design, required some maneuver and some force.

- Use baby powder to make insertion easier.

4) If the scope still will not advance, contact Technical Support.